Arduino IDE

Allows human readable data in the form of a console or a real time graph.

e.g. Temp:20, Humidity:80\n

Pros

Very popular

Allows for easy access to the specific things you’re interested in

Graphing is super simple to use.

All the examples tend to use it

Arduinos have the USB to Serial functionality built in

Cons

Needs to be programmed in by hand by the user

Provides no oversight on any of the other parts of the firmware operation

Graphing is limited in terms of no control over things like number of samples shown and scaling

No way (I’m aware of) of logging the data other then doing it manually.

Flowcode ICD

Allows for control over the program running on the microcontroller (start, stop, pause, breakpoints) and allows for real time monitoring and control of global variables (and with a bit more work registers).

Pros

Allows for access to most things, full oversight and control over the firmware

Icon by Icon resolution

All Global variables (and potentially registers) can be read and written.

Nothing is required of the user, they simply turn it on.

Cons

Bloats the firmware and changes the performance of the system.

Requires a man in the middle which goes between Flowcode and the MCU (Ghost IC).

The man in the middle approach is flawed, its difficult to work with, debug and proven to be unstable.

No graphing ability on variable values.

Idea1 - Flowcode Super ICD

What if there was a way to make a hybrid that could provide all the features of Flowcode ICD but without the man in the middle hardware. Instead of communicating with specific ICD hardware we instead talk directly to the MCU using the features it has onboard, e.g. USB, UART to USB, Bluetooth, Wifi, Ethernet. The various hardware can buffer short messages and therefore can be used to hold the ICD control signals until the firmware can jump in and process them.

We would need to define a format, maybe something fairly simple and then allow this to be done via Serial or network comms. This would need to be an EXE change but ideally could be farmed out to some type of external xml or plugin to give us more flexibility.

Pros

ICD Comms becomes a lot simpler to debug and more reliable using the peripherals on the MCU

It would work on just about any hardware and wouldn’t be dependent on E-blocks / Flowkit

It would be scalable and we could add more features to it over time

Due to no external hardware required would potentially allow for control and oversight over a program that’s been deployed

Cons

There would be a bit more bloat added to users programs to handle all the back and forth comms

It would require the assignment of a peripheral or component, so there would be a bit of setup on the users part, we could probably assume the default setup for things like Arduino/ESP32 boards

Not all of our E-blocks have a USB to Serial feature or other data connection currently.

Idea 2 - Flowcode Data Logging Component

On top of the super ICD we also have a component to allow a user to send data to a graph window in Flowcode (data recorder).

Pros

Very easy for the user to setup and get working

Allows for easy access and visualise to the specific things you’re interested in

Graphing is super simple to use.

We could provide log to file out of the box

Cons

Needs to be programmed in by hand by the user

Not all of our E-blocks have a USB to Serial feature or other data connection currently.