# Display Creator Technical Overview

# DC Primitive Components

Objects – screen elements with no touch interaction

Widgets – screen elements with touch interaction

DC_Hidden – creates a hidden area with a specified background colour to allow for correct text clearing. Available as an object or a widget.

DC_Rectangle – creates a rectangle with a specified foreground and background colour to allow for correct text clearing. The foreground colour forms a single pixel width border around the area. The background colour fills the remainder of the area. Available as an object or a widget and compatible with text. The widget has an additional drop shadow drawn in a third colour.

DC_RoundedRectangle – creates a rounded rectangle with a specified foreground and background colour to allow for correct text clearing. The foreground colour forms a single pixel width border around the area. The background colour fills the remainder of the area. Available as an object or a widget and compatible with text. The widget has an additional drop shadow drawn in a third colour.

DC_Ellipse – creates a circle or oval with a specified foreground and background colour to allow for correct text clearing. The foreground colour forms a single pixel width border around the area. The background colour fills the remainder of the area. Available as an object or a widget and compatible with text. The widget has an additional drop shadow drawn in a third colour.

DC_Line – creates a line with a specified foreground colour. The foreground colour forms a single pixel width line from the top left to the bottom right. Available as an object only and not compatible with text.

DC_Slider – creates a rectangle defining the touch area and bounds of the slider movement. A thin solid rectangle is drawn in the area representing the possible movement area. A rectangular or circular thumb with a drop shadow is drawn to indicate the current position of the slider widget. Available as a widget only and not compatible with text.

DC Bitmap – creates an area to draw a bitmap in a native or stretched resolution with transparency support. Available as an object or a widget and compatible with text.

Currently lines and borders are 1 pixel thick, ideally we could change this and allow for variable width lines.

We additionally need a way of drawing bitmaps and highlighting when a file is not found.

Also need to think about graphs and multi-line text fields.

Do we want sliders that can’t be touched? i.e. a slider object?

# Display Manager Primitive Encoding

## Object Encoding – 20 Bytes

- Byte: Type = ‘O’

- Byte: SubType = 0-4 / 0=Hidden, 1=Rect, 2=RoundedRect, 3=Ellipse, 4=Line

- Words: X, Y, Width, Height

- Byte: Line Width

- Byte x3: ForegroundColour

- Byte x3: BackgroundColour

- Byte x3: TextColour

## Widget Encoding – 26 Bytes

- Byte: Type = ‘W’

- Byte: SubType = 0-4 / 0=Hidden, 1=Rect, 2=RoundedRect, 3=Ellipse, 4=VSlider, 5=HSlider, 6=VSliderR, 7=HSliderR

- Words: X, Y, Width, Height

- Byte: Line Width

- Byte x3: ForegroundColour

- Byte x3: BackgroundColour

- Byte x3: TextColour

- Byte x3: HighlightColour

- Byte: Draw Highlight = 0-No / 1-Yes

- Word: Handle

## Bitmap Encoding – 18 Bytes + String Length + Null

- Byte: Type = ‘B’

- Byte: SubType = 0 – Object / 1 Widget

- Byte: Scaling = 0 – Native / 1 Stretch

- Words: X, Y, Width, Height

- Byte: Use Transparency = 0-No / 1-Yes

- Byte x3: Transparent Colour

- Byte x3: TextColour

- ByteArray: Bitmap Filename null terminated

## String Encoding – 4 Bytes + String Length + Null

Initialiser strings follow on from an Object or Widget allowing the returned ID to be used when adding the string data.

- Byte: Type = ‘S’

- Byte: Font Index

- Byte: Horizontal Alignment = 0-2 / 0=Left, 1=Centre, 2=Right

- Byte: Vertical Alignment = 0-2 / 0=Top, 1=Middle, 2=Bottom

- ByteArray: String Data null terminated

## Background Colour Encoding – 4 bytes

Used to set the background colour of the scene.

- Byte: Type = ‘C’

- Byte x3: Scene Background Colour

# Display Manager Macros

Here is a list of some of the important display manager component macros and the job they do.

## Prv_EncodeDCComps

Loops through all the linked scene panels and steps through all the components on each panel. Converts the panel components into binary data arrays which are stored in the Layout(X) properties. Automatically called as part of Ev_SimStart or Ev_CompileStart.

## Prv_FindSceneCompCallback

Provides the simulation method of obtaining and calling the specified callback macro. The embedded calls the callback macro via an array of function pointers.

## Prv_SetupDCScenes

Checks there are enough display creator panels and if required adds more and configures them.

## Prv_UpdateDCObjects

Loops through all the components on the top level and sends out a system.notify call which will flag a Ev_Property for all DC primitives. Updates things like fonts or schemes on the fly.

## Ev_AddHeader

Declares the callback macro type as an array of function pointers.

Adds the Layout(X) properties into the C code as ROM arrays.

Adds the callbacks to the C code to be parsed.

## DrawScene

Public macro used to collect and parse the data from the binary array. Calls things like Clear, CreateObject, CreateWidget etc to allow the contents of the scene to be drawn.

# Web Mirror MQTT Topics

The web mirror is controlled and operated via the use of the Matrix MQTT server and specific topic strings. The topic strings include a UID which comes from the user’s project and is formed by using the MAC address of the ESP32 module attached to the Matrix cap touch smart display.

## Embedded to Web Topics

### DC_UID_ST - Setup

This contains the setup data such as screen width and height and the fonts being used.

### DC_UID_SC - Scene

This contains the encoded data to drive the scene, it is static for as long as the scene is in use and it is automatically populated when the DisplayManager DrawScene function is called.

### DC_UID_AC - Action

This contains the blow by blow data as it is updated on the screen. Usually just containing text/graph data as it is changed. We will try a single item at a time and see if anything gets missed when lots of on screen elements are changing.

ID, Type, Variable Params

Type = 0 – String Label – STRING data, NUMBER Align (0x01 = Transparent, 0x06 = HALign, 0x18 = VAlign, 0xE0 = Font)

Type = 1 – Slider Value – FLOAT value

Type = 2 – Modify Position – NUMBER X, NUMBER Y, NUMBER Relative

Type = 3 – Modify Scale - NUMBER Width, NUMBER Height

Type = 4 – Modify Colour – NUMBER idx, NUMBER R, NUMBER G, NUMBER B

Type = 5 – Modify Bitmap – STRING filename, NUMBER stats (0x01 = Scaling, 0x02 = Transparency, 0x04 = Redraw)

Type = 20 – Setup – NUMBER MaxObjects, NUMBER MaxWidgets

## Web to Embedded Topics

### DC_UID_TC - Touch

This contains the X,Y coordinates of the last touch by the web user over the canvas field. E.g. a mouse click or touchscreen press. This is handled by the DisplayManager in the same way a press of the touchscreen would be handled.

# Binary Font Format

| Header |  |
| --- | --- |
| 0 | ‘F’ |
| 1 | ‘O’ |
| 2 | ‘N’ |
| 3 | ‘T’ |
| 4 | PixelWidth |
| 5 | PixelHeight |
| 6 | FirstChar |
| 7 | CharCount |
| 8 | DegreesChar |
| 9 | BytesPerColumn |
| 10 | UpperOnly |
| 11 | CharacterSpacing |
| 12 | SpaceWidth |
| 13 | AsciiStart0 |
| 14 | AsciiStart1 |
| 15 | NULL |
| Data |  |
| WidthDataArray[CharCount] - If PixelWidth = 0 |  |
| HeightDataArray[CharCount] - If PixelHeight = 0 – Not Yet Implemented |  |
| VOffsetDataArray[CharCount] - If PixelHeight = 0 – Not Yet Implemented |  |
| AsciiDataArray[AsciiLen] |  |